1. Objective

The objective of this project is to design and implement a touchless Human–Computer Interaction (HCI) system using an NVIDIA Jetson Nano that interprets real-time hand gestures and translates them into media control commands such as play/pause, volume control, mute, seek, and track navigation for a local media player (VLC).

The system aims to achieve the following goals:

- Low-latency real-time performance

- Robust gesture recognition under varying lighting conditions

- A contact-free interaction mechanism suitable for smart environments

In addition to gesture-based control, the system incorporates two advanced features to improve usability and security. A gesture locking/unlocking mechanism is implemented to prevent unintended command execution when gestures are detected unintentionally. Furthermore, a face authorization module is integrated to ensure that only registered users can access and control the system. These enhancements transform the system from a basic gesture interface into a controlled and secure interaction framework.

2. Project Description

This project utilizes MediaPipe Hands for real-time hand landmark detection optimized for ARM architecture and Jetson GPU acceleration. Instead of relying on a trained deep-learning gesture classifier, a rule-based gesture recognition approach is implemented by analyzing finger states and palm geometry derived from detected landmarks.

Recognized gestures are mapped to media control actions using:

- DBus (MPRIS interface) for VLC playback control

- xdotool for system-level keyboard and window management

To achieve real-time responsiveness on an edge device, several optimizations are applied:

- Multi-threaded camera capture using the V4L2 backend

- Frame skipping and reduced inference resolution

- CLAHE-based contrast enhancement for lighting robustness

- Jetson performance scaling using jetson_clocks

The system also supports timed (hold-based) gestures to reduce accidental activations and provides real-time visual feedback, including FPS, system status, and gesture progress indicators.

3.High-Level Workflow

- Camera captures live video stream

- Frames are resized and preprocessed

- Face detection and authorization is performed to verify user access

- MediaPipe detects hand landmarks

- Finger states are computed using geometric rules

- Gesture is classified using rule-based logic

- Corresponding VLC/media command is executed

- Visual feedback is displayed to the user

4. Hardware Requirements

- NVIDIA Jetson Nano Developer Kit

- USB Webcam

- Monitor, keyboard, and mouse

- Power supply (5V ⎓ 4A recommended)

5.Software Requirements

- JetPack OS (with CUDA support)
Required for GPU acceleration and optimized execution on NVIDIA Jetson Nano.

- Python 3.10.12

Core programming language used for system implementation.

- OpenCV  4.11.0
Used for video capture, preprocessing (CLAHE, resizing), and GUI rendering.

- MediaPipe Hands 0.10.18
Provides real-time hand landmark detection using a lightweight neural model.

- Face Recognition (dlib-based ResNet model)
Used for face detection and face encoding (128D embeddings) for user authentication.

- NumPy 1.26.4
Required for numerical operations, array handling, and image processing.

- VLC Media Player
Target application for gesture-based media control.

- DBus (MPRIS interface)
Enables direct communication with VLC for playback control (Play, Pause, Seek, Next, Previous).

- xdotool
Used for system-level window control and keyboard event simulation.

- Threading

Enables parallel camera frame acquisition for low-latency performance.

6. Gesture Design and Mapping

| Gesture | Finger Pattern | Action |
| --- | --- | --- |
| Open Palm | [1,1,1,1,1] | Play / Pause |
| Index Finger | [0,1,0,0,0] | Volume Up |
| Index + Middle | [0,1,1,0,0] | Volume Down |
| Fist | [0,0,0,0,0] | Mute |
| Thumb | [1,0,0,0,0] | Next / Previous |
| Thumb + Index | [1,1,0,0,0] | Seek Forward / Backward |
| Pinky Finger | [0,0,0,0,1] | Lock / Unlock System |
| Three Fingers | Sum = 3 | Maximize / Minimize VLC |

Direction-based gestures use handedness (left/right) for contextual interpretation.

7. System Design and Implementation

The system is designed as a modular, real-time HCI pipeline optimized for edge deployment on the NVIDIA Jetson Nano.

- Video Acquisition

A USB webcam captures frames using the V4L2 backend to minimize capture latency.

- Preprocessing

Frames are horizontally flipped for intuitive interaction, resized to 256×256 resolution, and enhanced using CLAHE to handle varying lighting conditions.

- Face Detection and Authorization

Face recognition is performed using a dlib-based ResNet model to identify authorized users. The input frame is downscaled for faster processing, and histogram equalization is applied to improve robustness. The detected face is encoded into a 128-dimensional feature vector and compared with stored encodings using a strict threshold. Only if the user is authorized, the system proceeds to gesture processing.

- Hand Landmark Detection

MediaPipe Hands detects 21 hand landmarks per frame using a lightweight neural model optimized for embedded platforms.

- Gesture Recognition

Finger states are computed using relative landmark positions and palm-size normalization to ensure scale invariance. Gestures are classified using deterministic rules.

- Command Execution

Recognized gestures are mapped to VLC media commands using DBus and xdotool, and are executed only when the system is authorized and unlocked.

- User Feedback

Real-time overlays display FPS, current gesture state, authorization status, and hold progress for improved usability.

8 . Mechanism

8.1 Gesture Execution Logic

- Instant Actions

Volume up and volume down commands are executed immediately to provide responsive control.

- Hold-Based Actions

Critical commands such as play/pause, mute, seek, and track navigation require the gesture to be held for a predefined duration to prevent false triggers.

- Cooldown Mechanism

A cooldown period is implemented after each command execution to avoid repeated or unintended activations.

- Gesture Lock/Unlock Mechanism

To avoid unintended or accidental gesture execution, a gesture locking mechanism is introduced.

- The system can be in two states:
Locked State: Gestures are detected but no commands are executed
Unlocked State: Gestures are actively mapped to system/media commands

- This mechanism ensures:

  - Prevention of false triggers

  - Better user control

  - Safe operation in continuous detection environments

This feature is particularly important in real-world scenarios where hands may remain in the camera frame without the intention to control the system.

8.2 Face Authorization Module

To introduce a security layer, a face recognition–based authorization system is integrated to restrict system access to authorized users only. If the user is not authorized, all gesture-based commands are disabled regardless of detection.

Model Used

- Library: face_recognition (based on dlib ResNet CNN model)

- Face Encoding: 128-dimensional embedding vector

- Detection Model:

  - HOG-based detector (real-time inference)

  - Optional CNN detector for higher accuracy during enrollment

Algorithm Workflow

- Face Detection

  - Performed using HOG-based model for real-time efficiency

  - Frame downscaling (0.5x) for faster processing

- Preprocessing

  - Histogram equalization (LAB color space)

  - Improves robustness under varying lighting

- Face Encoding

  - Each detected face is converted into a 128D feature vector

- Face Matching

  - Compared against stored authorized encodings

  - Uses Euclidean distance metric

- Decision Threshold

  - Tolerance = 0.40

  - Lower tolerance → higher security

9. Model Choice and Justification

9.1 MediaPipe Hands Model Selection

MediaPipe Hands was selected due to:

- Optimization for real-time inference on embedded systems

- Pre-trained robustness without the need for dataset collection

- High-precision landmark output instead of coarse bounding boxes

- Compatibility with ARM architecture and Jetson platforms

This model provides an effective balance between accuracy and computational efficiency.

9.2 Rule-Based Gesture Classification

A rule-based gesture recognition approach was chosen over a machine-learning classifier for the following reasons:

- Eliminates training complexity and dataset bias

- Provides deterministic and explainable behavior

- Faster execution with negligible computational overhead

- Easy tuning of gesture thresholds and sensitivity

10. Performance Optimization Techniques

- Multi-threaded camera capture

- Frame skipping to reduce inference load

- Reduced inference resolution (256×256)

- CLAHE-based contrast enhancement

- Jetson maximum performance mode

- Lightweight geometric gesture logic

- Face recognition executed every 15 frames to reduce computation load

- Frame downscaling for faster encoding

- Hybrid detection (CNN for enrollment, HOG for runtime)

- Time-based authorization caching to avoid repeated computation

11. Performance Analysis

11.1 Accuracy

- Average gesture recognition accuracy: >90%

- Stable hand detection across all supported gestures

- Minimal false positives due to hold-based confirmation logic

11.2 Latency

- Average inference latency: ~15–50 ms

- End-to-end gesture-to-action latency: <200 ms

11.3 Frame Rate

- Stable frame rate: 20–25 FPS

- Consistently above the minimum requirement of 15 FPS

11.4 System Stability

The system runs continuously without crashes or memory leaks. Cooldown mechanisms ensure stable behaviour during prolonged use.

12. Evaluation Against Performance Targets

| Metric | Target | Achieved |
| --- | --- | --- |
| Gesture Accuracy | >90% | Achieved |
| End-to-End Latency | <200 ms | Achieved |
| Frame Rate | ≥15 FPS | Achieved |

13. Results

The system successfully enables touchless media control with:

- Stable and accurate hand tracking

- Low-latency response

- Reliable gesture recognition

- Smooth integration with VLC media player

Real-time visual overlays improve transparency and user confidence during interaction.

14. Summary

This project demonstrates a robust, low-latency, touchless HCI system using NVIDIA Jetson Nano. By combining MediaPipe Hands with a carefully designed rule-based gesture engine and system-level optimizations, the solution meets all defined performance targets and is suitable for real-world edge AI deployment.